Calibration method of the magnetic/velocity data obtained by T1 and T4

K.Ichimoto, 1992/10/28

1992/12/02  Rev.

1993/11/16  Rev.

1. Theoretical calculations of the line formation

Purpose:

Obtaine the relations of

for FeI6303A;

V     <--> Bl

Q,U   <--> Bt

with the 1/8 A Lyot filter.

for FeI6337A;

V     <--> Bl & D

S     <--> D  & Bl

with the 1/5 A Lyot filter.

where Bl is longitudinal  and Bt is transversal magnetic field,

S is Doppler signal and D is Doppler shift.

Programs:

/home/spot/ichimoto/transfer/mcalib.c       (main)

/              "            /lib/ltelib.c   (libraly)

/              "            /lib/atmos.c    (   "   )

/              "            /lib/lines.c    (   "   )

/              "            /lib/transfer.h (include)

Method:

LTE + I,Q,U,V transfer including the magneto-optical effect.

Atmosphere:

Holweger,H. & Muller,E.A. 1974, SP.39,19 standard atmosphere (58 step),

with uniform magnetic fields.

Line parameters:

The profile is adjusted to fit the Liege Atlas

under the no magnetic field atmosphere.

FeI6303A;

central wave length  = 6302.499

excitation potential = 3.69 (eV)

log(gf)              = -1.50

dumping constant     = 0.3

Lande's geff	     = 2.5

FeI6337A;

central wave length  = 6336.830

excitation potential = 3.69 (eV)

log(gf)              = -1.25

dumping constant     = 0.3

Lande's geff	     = 2.0

Calculation parameters:

cos(th) = 1       ; line of sight it normal to the surface.

wavelength points ; -0.4 ~ 0.4 A, 201 points.

magnetic field    ; vertical or horizontal in x-direction

step;  0 ~ 3000 Gauss, 301 points.

Results:

I,Q,U,V profiles are saved in

/home/ichimoto/transfer/mcalib.dat/a6303v.norm (normal-B)

/       "                         /a6303q.norm (horizontal-B)

/       "                         /a6337v.norm (horizontal-B)

These data can be read by

IDL> mcalget,file,wl,i,q,u,v,hmag,l,info

Display P-B & S-Vel relations:

IDL programs  -- /home/ichimoto/transfer/idlpro/*.pro

IDL> .run mcalp03     after editting it,

shows P-B relation for 16 wavelength position of the 1/8A Lyot filter.

IDL> .run mcalp37     after editting it,

shows S-Vel relation for 5 magnetic field strength.

2. Making calibration table from the theoretical calculations.

IDL> .run mkcaldat

makes followings;

for T1:

# V => Bl table at 16 wavelength.     ==> VBl1.dat

# Q => Bt table at 16 wavelength.     ==> QUBt.dat

for T4:

# V => Bl table at 16 wavelength.     ==> VBl4.dat

# S => Vel table at 31 mag. strength. ==> SVel.dat

where *.dat are made in '/usr/local/lib/idl/lib/flare'.

Each data file containes following array;

VBl1.dat			    QUBt.dat

-150mA _____________________   -150mA __________________

|                    |         |                 |

dwl |     Bl (G*10)      |     dwl |    Bt (G*10)    |

0 |____________________|	   0 |_________________|

-4000      V*10000     4000       0      Q*10000    4000

(8001-points)		(4001-points)

VBl4.dat			    SVel.dat

-150mA _____________________    3000G ____________________

|                    |         |                   |

dwl |     Bl (G*10)      |     Bl  |     Vel (m/s)     |

0 |____________________|	  0  |___________________|

-4000      V*10000     4000    -4000      S*10000    4000

(8001-points)		(8001-points)

These data can be read by

IDL> getcalib1,Bl,Bt,dwl=dwl,/extra

IDL> getcalib4,Bl,Vel,dwl=dwl,bmag=bmag,/extra

where dwl is the wavelength position of the Lyot filter in mA

and bmag is the magnetic field strength in Gauss.

If /extra is assigned, calibration curves are extraporated beyond

the saturation level by linear relation.

Translation can be done by

Bl = Bl(v+4000)

Bt = Bt(pt)	where Pt = sqrt(q^2+u^2)

Vel= Vel(s+4000)

3. Getting data

for T1 data:

'f1a##.v', 'f1a##.q', 'f1a##.u' contain I,V, I,Q, I,U images.

Dark and flat corrections were made already.

'f1a##.*.p' or 'fbyymmdd.hhmm.p' contain I,Q,U,V images,

where Q,U,V are the ratios of the subtraction image

of 2-polarization and intensity I, which multiplied by 20000.

Displacements of sunspot positions in Iq,Iu,Iv are corrected.

To get those images,

IDL> f1data,'f1a##',h,img,q,u,v

or  IDL> f1data,'fbyymmdd.hhmm.p',h,img,q,u,v

or  IDL> nkrget,'fbyymmdd.hhmm.p',h(0),img,img_no=1

IDL> nkrget,'fbyymmdd.hhmm.p',h(1),q,img_no=2

IDL> nkrget,'fbyymmdd.hhmm.p',h(2),u,img_no=3

IDL> nkrget,'fbyymmdd.hhmm.p',h(3),v,img_no=4

or  IDL> .run fdisp

for T4 data:

'f4a##.v', 'f4a##.s' contain I,V, I,S images.

Dark and flat corrections were made already.

'f4a##.*.p' or 'fvyymmdd.hhmm.p' contain I,S,V images,

where S,V are the subtraction images of 2-wavelength and

2-polarization devided by intensity I and multiplied by 20000.

Displacements of sunspot positions in Is,Iv is corrected.

To get those images,

IDL> f4data,'f4a##',h,img,s,v

or  IDL> f4data,'fvyymmdd.hhmm.p',h,img,sv

or  IDL> nkrget,'fvyymmdd.hhmm.p',h(0),img,img_no=1

IDL> nkrget,'fvyymmdd.hhmm.p',h(1),s,img_no=2

IDL> nkrget,'fvyymmdd.hhmm.p',h(2),v,img_no=3

4. Calibration procedures

Scattered lilght correction:

IDL> scatter,scatter,h,img,q,u,v

or IDL> scatter,scatter,h,img,s,v

where scatter is scattered light level which is assumed to be uniform.

Translations are;

img  --> img-scatter,

v    --> v*img/(img-scatter).

KDP error correction:

correction of KDP departure from quater wave plate.

just q => q*kdpfact,  u => u*kdpfact,  v => v*kdpfact

Derive the vector magnetic fields from Q,U,V (T1):

IDL> quv2b,q,u,v,Bx,By,Bl,dwl=dwl,Bl=Bl_cal,Bt=Bt_cal

where;

Bx,By,Bl are transversal and longitudinal magnetic field strength

in Gauss*10.

dwl gives the wavelength position of the Lyot filter in mA

(default=-80),

Bl_cal,Bt_cal  give/get the calibration tables.

Direction of Bx and By is determined refering to the potential field

calculated from Bl by the procedure 'cff1n.pro'(T.Sakurai).

Derive the magnetic field and velocity from S,V (T4):

IDL> sv2vb,s,v,Vel,Bl,bmag=bmag,dwl=dwl,Bl=Bl_cal,Vel=Vel_cal

where;

Vel is velocity in m/s.

Bl is longitudinal magnetic field strength in Gauss*10.

bmag gives the magnetic field strength in G (default=0),

dwl gives the wavelength position of the Lyot filter in mA

(default=0),

Bl_cal,Vel_cal  give/get the calibration tables.

Standards of the calibration parameters

scatter = (1-0.7)*imin*imax/(imax-0.7*imin)

for imin/imax > 0.17,  disk image

= imin		for imin/imax < 0.17,  limb image including sky

kdpfact = 1.05

dwl = -80 mA for T1

0 mA for T4

Determin scattered light level interactively:

IDL> scatter=sctlevel(img)

chose a method from following

* assume spot darkness as 15% of normal region

* use sky bringhtness

* enter scatter level relative to normal region

* enter scatter level

If determined scatter < min(img), scatter is replaced by min(img).

5. Calibration of vector mag.field data of T1

IDL> f1calib,img,q,u,v,bx,by,bl,h=h,scatter=scatter,dwl=dwl,	$

kdpfact=kdpfact,Bl_cal=Bl_cal,Bt_cal=Bt_cal

where:

Inputs are;

img	-- image

q,u,v   -- pol. degree *20000 (q,u,v already devided by img)

Outputs are;

img	-- modified image for scatter

bx,by,bl - magnetic field (transversal & long.) in Gauss*10

Keywords are;

dwl -- wave length position of Lyot filter from line center (mA)

if dwl is not specified, -80 mA is assumed

scatter -- sacttered light intensity

if not set, scatter=imin     (imin/imax < 0.17, limb image)

=(1-0.7)*imin*imax/(imax-0.7*imin)

(	   > 0.17, disk image)

kdpfact	-- kdpfactor, default=1.05

h 	-- header structure for modification

Bl_cal  -- calibration function of Bl,    p(*10000) => B(G*10)

Bt_cal  -- calibration function of Bt,    p(*10000) => B(G*10)

IDL> f1calibf,filenames,dwl=dwl,scatter=scatter,kdpfact=kdpfact, $

dir=dir,outfiles=outfiles,bin=bin

reads data from 'fbyymmdd.hhmm.p' (for ex.),

makes above corrections and calibration,

modify headers,

and save the results in 'fbyymmdd.hhmm' (if outfiles is not specified)

in order of img,Bx,By,Bl.

The default of parameters are;

dwl=-80,

scatter= (above procedure),

kdpfact=1.05,

bin=1 (no binning).

Calibration information are recorded in headers as follows:

h.value   =>   I,   Bx,  By,  Bl

h.correct =>  101, 101, 101, 101  <--- calibration version

h.dx	  =>  pix1*bin

h.unit    =>      'G*10' "    "

h.com     =>  'f1calib, dwl=##   Mon dd hh:mm:ss'

h.d_parm(10)  =>  dwl

h.d_parm(11)  =>  scatter

h.d_parm(12)  =>  kdpfact

6. Calibration of mag./velocity fields of T4

IDL> f4calib,img,s,v,vel,bl,h=h,Bl_cal=Bl_cal,Vel_cal=Vel_cal,	$

scatter=scatter,kdpfact=kdpfact,dwl=dwl,bmag=bmag

where:

Inputs are;

img	-- image

s,v     -- Dopp.signal & pol. degree *20000 (devided by img)

Outputs are;

img	-- midified image for scatter

bx,by,bl - magnetic field (transversal & long.) in Gauss*10

Keywords are;

dwl -- wave length position of Lyot filter from line center (mA)

if dwl is not specified, 0 mA is assumed

scatter -- sacttered light intensity

if not set, scatter=imin     (imin/imax < 0.17, limb image)

=(1-0.7)*imin*imax/(imax-0.7*imin)

(	   > 0.17, disk image)

kdpfact	-- kdpfactor, default=1.05

bmag    -- magnetic field strength in Gauss, default=0

h 	-- header structure for modification

Bl_cal  -- calibration function of Bl,    p(*10000) => B(G*10)

Vel_cal -- calibration function of Vel,   s(*10000) => Vel(m/s)

IDL> f4calibf,filenames,bmag=bmag,dwl=dwl,scatter=scatter, $

kdpfact=kdpfact,dir=dir,outfiles=outfiles,bin=bin

reads data from 'fvyymmdd.hhmm.p' (for ex.),

makes above corrections and calibration,

modify headers,

and save the results in 'fvyymmdd.hhmm' (if outfiles is not specified)

in order of img,Vel,Bl.

The default of parameters are;

dwl=0,

bmag=0,

scatter= (above procedure),

kdpfact=1.05,

bin=1 (no binning).

Calibration information are recorded in headers as follows:

h.value   =>   I,  Vel,  Bl

h.correct =>  101, 101, 101  <--- calibration version

h.dx	  =>  pix1*bin

h.unit    =>      'm/s' 'G*10'

h.com     =>  'f4calib, dwl=##  bmag=bmag  Mon dd hh:mm:ss'

h.d_parm(10)  =>  dwl

h.d_parm(11)  =>  scatter

h.d_parm(12)  =>  kdpfact

h.d_parm(13)  =>  kdpfact2

h.d_parm(14)  =>  bmag

Main procedure for all calibrations:

IDL> .run ftcal

Calibration parameters and files should be assigned in the program.

7. Display magnetic field map

IDL> magtv1,img,q,u,v,h=h,magnify=magnify,bin=bin

or	IDL> magtv1,img,Bx,By,Bl,/cal,h=h,magnify=magnify,bin=bin

or	IDL> magtv2,img,q,u,v,h=h,magnify=magni##########################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################